for do-self repairs

# YEARLY HOME MAINTENANCE SCHEDULE

The following is HRRC’s suggested schedule for home maintenance inspections and repairs:

# SPRING (April - June)

- Clean gutters; make sure they are flowing properly.

- Inspect roof for damaged shingles.

- Check flashing, and re-seal as needed.

- Renail any loose siding or trim boards.

- Check house and garage for any rotting wood, and repair as needed.

- Check porch flooring, and repair as needed.

- Check and replace any bad caulking around windows, doors, siding joints, or anywhere else needed.

- Scrape loose window glazing, and replace as needed.

- Clean windows and their frames when installing screens. Wash and repair screens at this time.

- Check caulk or seal between house and drive, and repair as needed.

- Oil garage door tracks.

- Trim trees, bushes, and shrubs. (Wait until spring-flowering shrubs have finished blooming.)

- Clean flower beds out around the house.

- Drain off sediment from hot water tank and steam heating system.

- Add copper sulfate to basement floor drain and downspout drains to control tree roots in main sewer.

# SUMMER (July - August):

- Scrape, prime, and paint any areas on house or garage that are in need, especially any bare wood. (Note: Observe precautions for reducing exposure to lead-based paint.)

- If the house or garage doesn't need paint, wash exterior off at least once.

- Look for visible signs of carpenter ants, termites, or any other destructive insects.

- Check all flat roofs, and recoat or seal as needed, especially flashing.

- Check for mortar that needs repair (including chimney mortar and inside fireplace,) and tuckpoint or repair as needed.

- Check concrete drive, sidewalks, patios, and landings for condition and level; patch cracks, level, and/or replace, as needed.

- Clean carpets and wax tile and hardwood floors.

- Add copper sulfate to basement floor drain and downspout drains to control tree roots in main sewe

(continued)

# FALL (September - October):

- Patch and seal driveway (asphalt or concrete), and concrete steps/ landings.

- Check tuckpointing on all brick areas, then seal masonry with a silicone sealant.

- Check weatherstripping on all doors and windows, and repair as needed.

- Check all areas that need to be caulked, and repair as needed.

- Make sure all garage floor or driveway drains are flowing properly.

- Clear gutters of leaves, check for leaks, and repair as needed.

- Inspect roof for loose or missing shingles, and check flashing seals; repair as needed.

- Clean and reseal decks, fences, or other structures made of pressure-treated wood.

- Oil garage door tracks.

- Prune bushes, trees, and shrubs (except spring-flowering shrubs.)

- Have furnace checked or tuned up, and change filter; have boiler checked every few years (every year if boiler

is more than 20 years old.

- Have chimney cleaned (yearly, if wood burning; every 5 years if just for gas appliances.)

- Check damper in fireplace for obstructions, such as birds' nests.

- Drain off sediment from hot water tank and steam heating system.

- Add copper sulfate to basement floor drain and downspout drains to control tree roots in main sewer.

# WINTER (November - March):

- At beginning of winter, shut off interior valves supplying outdoor water pipes to prevent them from freezing.

- Change furnace filters monthly.

- Repair any interior wall damage; touch up or repaint as needed.

- Check and re-grout tile in all bath and kitchen areas, as needed, and apply a silicone sealer.

- Add copper sulfate to basement floor drain and downspout drains to control tree roots in main sewer.

- Develop plan for next summer's home improvements.

- At winter’s end, re-open interior valves supplying outdoor water pipes.

©2011, Home Repair Resource Center, 2520 Noble Rd., Cleveland Hts., OH 44121 (phone: 216-381-9560)